Affordable, Reliable Lab Solutions

M.Wave professional

PC controlled Spectrophotometer Software

User’s Manual

Version 1.1

Abbota Corporation

178 Franklin Road

Randolph, NJ 07869

United States

Contents

Function at a Glance ………………………………………………………………………	1

Main Function …………………………………………………………………………..	1

Quantitative Analysis …………………………………………………………………	1

Kinetics ………………………………………………………………………………...	1

Wavelength Scan ……………………………………………………………………..	1

Multi-wavelength Test ………………………………………………………………...	1

DNA/Protein Test ……………………………………………………………………..	1

Install Software ……………………………………………………………………………. 	2

PC System Requirement ……………………………………………………………….	2

Install M.Wave Professional ……………………………………………………………	2

Uninstall M.Wave Professional ………………………………………………………...	3

Run M.Wave Professional …………………………………………………………….	3

Set Communication Port ………………………………………………………………..	4

Set User’s Information …………………………………………………………………..	4

Connect to/Release PC …………………………………………………………………	5

Software Introduction ………………………………………………………………………	6

Main Interface ……………………………………………………………………………	6

Menu Bar & Tool Bar …………………………………………………………………….	6

Operation ……………………………………………………………………………………	9

Quantitative Test …………………………………………………………………………	9

Measure …………………………………………………………………………………	9

Modify a Test Result ………………………………………………………………… 	11

Delete a Test Result ………………………………………………………………….	11

Name a Sample ………………………………………………………………………	11

Kinetics ………………………………………………………………………………… 	11

Measure a Sample …………………………………………………………………...	11

Switch Display Mode …………………………………………………………………	12

Magnify the Selected Zoon …………………………………………………………..	12

Rescale the Coordinates …………………………………………………………… 	12

Switch the Display status …………………………………………………………….	13

Search Peaks ………………………………………………………………………....	13

Wavelength Scan ………………………………………………………………………	13

scan ……………………………………………………………………………………	13

Switch the Display Mode …………………………………………………………… 	15

Magnify the Selected Zoon ………………………………………………………… 	15

Rescale the Coordinates …………………………………………………………… 	15

Switch the Display status …………………………………………………………… 	16

Search Peaks ………………………………………………………………………... 	16

Math Functions …………………………………………………………………….. 	17

Example 4-1 ………………………………………………………………………. 	17

Spectrum Smooth …………………………………………………………………….	18

Multi-wavelength Measurement ………………………………………………………	18

Measure ……………………………………………………………………………… 	18

Modify a Test Result ………………………………………………………………… 	19

Delete a Test Result ………………………………………………………………… 	19

Name a Sample ……………………………………………………………………… 	20

Switch Display Mode ………………………………………………………………... 	20

DNA/Protein Analysis ………………………………………………………………… 	20

Measure ……………………………………………………………………………… 	20

Modify a Test Result ………………………………………………………………… 	21

Delete a Test Result ………………………………………………………………… 	21

Name a Sample ……………………………………………………………………… 	21

Other Function …………………………………………………………………………… 	22

Set Wavelength ……………………………………………………………………….. 	22

Setup Switch Point of Light Source …………………………………………………. 	22

Turn On/Off W Lamp …………………………………………………………………. 	22

Turn On/Off D2 Lamp ………………………………………………………………… 	22

Set Light Source Switch Mode ………………………………………………………. 	22

Set Response Mode ………………………………………………………………….. 	22

Get Dark Current ……………………………………………………………………… 	23

File Process ……………………………………………………………………………… 	24

Save Test Result ……………………………………………………………………… 	24

Open a File ……………………………………………………………………………. 	24

Print Test Report ……………………………………………………………………… 	24

Appendix 1 ……………………………………………………………………………….. 	25

Method of Quantitative Analysis …………………………………………………….. 	25

Appendix 2 ……………………………………………………………………………….. 	26

DNA/Protein Analysis Method ………………………………………………………. 	26

#### Function at a Glance

We’ll introduce you the main functions of M.Wave Professional in this chapter。

Main Function

Quantitative Analysis

We provide you 2 methods to establish a Standard Curve:

·Coefficient Method

If you have known the coefficients of a curve’s formula, you can input them directly, then the curve displays on the screen. You can measure unknown samples using the curve.

·Create a curve by standard samples

You can use at most 20 standard samples to create a new curve. 3 ways are under your choice to fit the curve, which are Linear, Square and Cubic ways.

Kinetics

·You can choose the Time Intervals(0.5, 1.0, 2.0, 5.0, 10.0, 30.0 or 60.0 Seconds).

·You can choose different Photometric Mode to display the curve. (Transmittance-Time & Absorbance-Time)

Wavelength Scan

·You can save and load the System Base Line.

·You can choose the Scan Intervals(0.1, 0.2, 0.5, 1.0, 2.0 and 5.0nm).

·You can choose the Photometric Mode to display the spectrum. (Wavelength-Transmittance, Wavelength-Absorbance or Wavelength-Energy)

·Automatically list spectrum peaks.

·Spectrum process(+,-,*,/,derivative and smooth)

Multi-wavelength Test

·You can set at most 20 wavelengths to measure a sample.

DNA/Protein Test

·Two methods are under your choice.

·You can input the coefficients by yourself.

#### Install Software

We’ll show you how to install M.Wave Professional on you PC in this chapter.

PC System Requirement

· Pentium or above PC

· CD-ROM

· USB Port

· 32 MB Memory(256 MB or Above is strongly recommended）

· 50 MB or above Hard-disc Space

· Microsoft Windows® 2000 or Windows® XP

Install M.Wave Professional

· Put M.Wave Professional disc in the CD-ROM.

· Open the files on the disc.

· Double Click Setup.exe to begin the installation，a welcome interface will appear on the screen (Fig.2-1). Click Next.

· Input the user’s information, then click Next.（Fig. 2-2）.

· Choose install path, then click Next（Fig. 2-3）.

· Choose Shortcut file folder, then click Next Step.（Fig. 2-4）. All the setting information will be displayed on the screen(Fig.2-5）, click Next, all the files will be copied to the Hard Disc.

· A USB setup interface (Fig.2-6) will appear after last step. Click Install to install USB driver. Click OK to confirm in the new frame. (Fig.2-7)

· Click Finish to finish the installation. (Fig.2-8)

Fig. 2-1 Welcome Interface                           Fig. 2-2 User’s Information

Fig. 2-3 Install Path                             Fig. 2-4 Choose Shortcut Folder

Fig. 2-5 Display User’s Information                      Fig.2-6 Install USB Driver

Fig.2-7 Finish Install USB Driver                      Fig. 2-8 Finish Total Install

Uninstall M.Wave Professional

Please find the Uninstall file in the listed path on your PC’s desktop.

·Start→ All Files→ M.Wave Professional→ Uninstall M.Wave Professional 1.0， Click the last menu, then click Next to Uninstall.(Fig. 2-9）

·After all the files are removed，click Finish to end. (Fig. 2-10）

Fig. 2-9 Uninstall Information                       Fig. 2-10 Finish Uninstallation

Run M.Wave Professional

Two ways are under your choice to run M.Wave Professional.

- Double click the icon  on your PC’s desktop.

- Start→ All files→ M.Wave Professional→ M.Wave Professional1.0.

If the listed information (Fig. 2-11) appears, click OK first, then you should insert the USB Key to the PC’s USB Port and restart the software.

Fig.2-11 Did not find Softdog dongle.

Setup Communication Port

After M. Wave Professional being started，click View then Option on the Menu Bar. Click Search to find the communication port in the following frame(Fig 2-12），then click OK to confirm(Fig.2-13). Click the icon   in the Tools Bar to connect to PC.

Fig.2-12 Search the communication port

If you choose the option of Connect to Spectrophotometer When Start，then the next time you run the software, the instrument will connect to the PC automatically.

Fig. 2-13 Setup Communication Port

Set User’s Information

Click User’s Information in Fig.2-13, then Input your information, click OK to confirm. (Fig. 2-14)

Fig.2-14 Set User’s Information

Connect to or Release PC.

Click the icon   in the Tools Bar to connect to or release PC.

If you can find the model name of the instrument in the lower corner of the screen’s Left area (Status Bar), then the instrument connects to the computer successfully. Or it displays Unconnected in this area. (Fig.3-1)

#### Software Introduction

We will introduce you the total running environment of M.Wave Professional in this chapter.

Main Interface

This is the main interface after start. (Fig. 3-1).

Fig. 3-1 Main Interface

Menus Bar and Tools Bar

Menus Bar and Tools Bar provide two different ways to operate this software.

·You can finish all the operation through Keyboard or Mouse.

·Most of the functions can be finished through the Shortcut Bar.

| Main Menu | Class1 Submenu | Class 2 Submenu | Tool Bar | Function |
| --- | --- | --- | --- | --- |
| File | New | Quantitative Analysis |  | Start a New Quantitative Test |
|  |  | Kinetics Analysis |  | Start a New Kinetics Test |
|  |  | Wavelength Scanning |  | Start a New Wavelength Scan Test |
|  |  | Multi wavelength Analysis |  | Start a New Multi Wavelength Test. |
|  |  | DNA/Protein Analysis |  | Start a New DNA/Protein Test |
|  | Open |  |  | Load a Saved File |
|  | Close |  |  | Close the Current Test |
|  | Save |  |  | Save the Current Test |
|  | Print |  |  | Print Test Report |
|  | Exit |  |  | Exit M.Wave Professional |
| View | Tools bar | Standard |  | Display/Hide Tools Bar |
|  |  | Spectrum |  | Display/Hide Spectrum Process Tools Bar |
|  | Status bar |  |  | Display/Hide Status Bar |
|  | Options |  |  | Set System Parameters |
| UV-Spectrophotometer | Connect/Release |  |  | Connect to or Release the PC |
|  | Turn On/Off W Lamp |  |  | Turn On/Off W Lamp |
|  | Turn On/Off D2 Lamp |  |  | Turn On/Off D2 Lamp |
|  | Setup Switch Point of Light Source |  |  | Set Wavelength Point at Which the Lamps Switch |
|  | Light Sources Switch Mode | Auto Switch Mode |  | Switch Automatically at the Setting Wavelength Point |
|  |  | D2 Lamp Mode |  | Use D2 In All the WL. Range |
|  |  | W Lamp Mode |  | Use W Lamp In All WL. Range |
|  | Setup System Baseline |  |  | Build System Baseline |
|  | Response Mode | Standard |  | Scan at the Standard Speed |
|  |  | Fast |  | Quick Scan |
|  | Get Dark Current |  |  | To Get Dark Current Again |
|  | Goto Wavelength |  |  | Set Wavelength |
| Operation | Start |  |  | Test |
|  | Cancel |  |  | Cancel Current Test |
|  | Modify |  |  | Re-measure the Current Sample |
|  | Delete |  |  | Delete Current Test Data |
|  | Zero/Full |  |  | Set Zero and 100%T |
|  | Setting |  |  | Set Test Parameters |
| Spectrum | Back |  |  | See the Spectrum’s Upper Status |
|  | Forward |  |  | See the Spectrum’s Latter Status |
|  | Display Mode | Abs. |  | Set the Photometric Mode to Abs. |
|  |  | Tran. |  | Set the Photometric Mode to Trans. |
|  | Zoom | Zoom Range |  | Reset Scales of the Coordinate |
|  |  | Zoom Window |  | Magnify the Selected Area |
|  |  | Zoom All |  | Resume the Original Status |
|  | Peak |  |  | Auto Search the Spectrum Peak |
|  | Height Setup… |  |  | Set the Height of the Peak |
|  | Math(t) | Addition… |  | Add Two Spectrums |
|  |  | Subtraction… |  | Subtract One Spectrum from Another |
|  |  | Multiplication… |  | Multiply Two Spectrums |
|  |  | Division… |  | Divide One Spectrum from Another |
|  |  | Derivative… |  | Derivative of a Spectrum |
|  | Smooth |  |  | Smooth a Spectrum |
| Window | New |  |  | Create a New Measurement Window as Current. |
|  | Close |  |  | Close the Current Test |
|  | Cascade |  |  | Multi Windows Display in a Cascade |
|  | Tile Vertically |  |  | Multi Windows Display in a Vertically Tile |
|  | Tile Horizontally |  |  | Multi Windows Display in a Horizontally Tile. |
| Help | About… |  |  | Information About M.Wave Professional |

Table 3-1 Menus and Shortcut Icons

#### Operation

We will introduce you how to use M.Wave Professional。

Quantitative Test

M.Wave Professional use Standard Curve to test the sample’s concentration at a fixed wavelength point.

Measure

1. Click the icon  in the Tools Bar to create a new Quantitative Test. (Fig. 4-1)

Fig. 4-1 the Main Interface of Quantitative Test

2. Click the icon  in the Tools Bar to set the parameters of Quantitative Test. (Fig. 4-2

D

)

A

B

C

E

F

Fig. 4-2 Parameters Setting Interface of Quantitative Test

3. Select Test Method in Area A (Fig.4-2)  	Pls. Ref. Appendix 1

4. Input the wavelength you want in Area B (Fig.4-2).

5. Choose the Concentration Unit in Area C (Fig.4-2).

6. Choose curve fitting method in Area D (Fig.4-2)

7. Create Standard Curve.

Two methods are under your choice to create a new curve.

·Method 1：Coefficient

(1) Click Coefficient option in Area E. (Fig.4-2)

(2) Input the curve equation’s coefficients in corresponding frames in Area E. (Fig. 4-2).

(3) Click OK to finish setting.

·Method 2：Use Standard Samples to create a new curve

(1) Click Standard Sampling option in Area E (Fig.4-2).

(2) Choose the quantity (≤20)of the standard samples in Area E. (Fig.4-2)

(3) Input the standard samples’ concentration in corresponding frame in Area F. (Fig.4-2)

(4) Click OK to finish setting.

(5) Pull the Reference into the light path and click the icon ，then the system will move the wavelength to the one you just set, it blanks then.

(6) Pull 1# standard sample into the light path and move the cursor on the first frame of Abs. in Area C. (Fig.4-3) The sample’s Absorbance value will appear in this frame after double click.

(7) Measure other standard samples in the same way as step (6) shows. The curve displays automatically in Area B (Fig.4-3) when finish.

B

C

A

Fig. 4-3 Create Standard Curve and Measure

Note: You can modify the curve’s coordinates by clicking the icon . (Fig. 4-4).

You can also save the curve by clicking   , and you can reload it by clicking   next time.

Fig. 4-4 rescale the Curve’s Coordinates

- Pull the reference into the light path, click the icon  again to set Zero and Full scale. Pull the unknown concentration sample into the light path, click the icon   in the tools bar to test. And the test result will display in the data sheet in Area A. (Fig.4-3)

- Finish all the test following the way as step 8 does.

Modify a Test Result

- Move the cursor on the data you want to amend and click.

- Pull the sample into the light path.

- Click the icon  in the Tools Bar to re-measure the sample, and the new value will replace the old one.

Delete a Test Result

- Move the cursor on the data you want to delete and click.

- Click the button  in the Tools Menu to delete the data.

Name a Sample

- Move the cursor on the sample line you want to name.

- Input the sample name after double click the frame, then press Enter to confirm.

Kinetics

We’ll introduce you how to measure a sample’s change of Absorbance or Transmittance over a selected period of time.

Measure a Sample

- Click the icon   to begin a new test of Kinetics.（Fig.4-5）.

Fig.4-5 Main Interface of Kinetics

- Click the button of   to set the parameters (Fig.4-6）

Fig.4-6 Set Parameters of Kinetics

- Choose Photometric Display Mode，input the wavelength, Delay Time, Total Time, Intervals and coordinates values

- Click OK to finish and cancel setting.

- Pull the reference into the light path and click icon   in the Tools Bar to set zero and full scale.

- Pull the sample into the light path and click the button of   to begin the test.（Fig. 4-7），you can click icon  in the Tools Bar to cancel at any time.

Fig. 4-7 Kinetics

Switch the Display Mode

Click the icon  or  in the Tools Bar to choose the photometric mode you want. (Abs.-Time, Tran.-Time)

Magnify the Selected Zoon

Click the icon  in the Tools Bar, choose the area you want to magnify, then you’ll see the magnified one, and the Mouse’s cursor turns into the form of “ + ”. Click the icon  again to cancel the current status.

Rescale the coordinates

Click the icon   in the Tools Bar, input the new value you want in the new frame (Fig. 4-8) and click OK to confirm. Then the curve with new coordinates will display on the screen.

Fig.4-8 Rescale the Coordinate

Switch the Display Status

Click the icon  or  to display the curve’s upper or latter status, click  to recover the original status.

Search Peaks

Click the icon  in the Tools Bar to search the peaks, they will be labeled automatically on the curve, and the peaks’ height will be listed in the Right Frame. (Fig.4-7)

Click the icon  in the Tools Bar. You should input the peaks’ height that you want to find in the new frame and click OK. (Fig.4-9) Then the peaks whose height is lower than the inputted value will be automatically labeled.

Fig.4-9 Set peak’s height

Wavelength Scan

We’ll introduce you how to create a spectrum while using Wavelength Scan function.

Scan

1)  Click the icon  in the Tools Bar to start a new wavelength scan test. (Fig.4-10)

Fig.4-10 the main interface of wavelength scan

  - Click the icon  in the Tools Bar to set the parameters of Wavelength Scan.(Fig.4-11)

Fig.4-11 Set parameters of wavelength scan

  - Set parameters

- Choose the photometric display mode 
Note: you can set the Amplifier when you choose Energy.

- input the wavelength range for scanning and the coordinates

- Choose scan step.

- Click OK to confirm and exit setting.

  - Click the icon  in the Tools Bar, then the dialog box of System Baseline displays in the screen. (Fig.4-12) If you want to use a baseline saved before, you can click Open to load, or you have to Input the values of start wavelength and end wavelength, then click Scan to build a new system base line.

Note:

    - You can click Cancel to interrupt and exit the course.

    - If you choose the option of Save System Baseline, another dialog box will appear on the screen to prompt you to save the baseline when finish.

  - Pull the Reference into the light path, click the icon    to set user’s baseline in the new dialog box. (Fig.4-13) You can also interrupt and exit the course by clicking Cancel.

Fig.4-12 Set system base line                      Fig.4-13 Set user’s base line

  - Pull the sample into the light path and click the icon  in the Tools Bar to start scanning (Fig.4-14). If you want to interrupt the course, you can click the icon .

Fig.4-14 Wavelength Scan

Switch the Display Mode

Click the icon  or  in the Tools Bar to choose the photometric mode you want. (Abs.-Wavelength, Trans.-Wavelength)

Magnify the Selected Zoon

Click the icon  in the Tools Bar, choose the area you want to magnify, then you’ll see the magnified one, and the Mouse’s cursor turns into the form of “ + ”. Click the icon  again to cancel the current status.

Rescale the coordinates

Click the icon   in the Tools Bar, input the new value you want in the new frame (Fig. 4-15) and click OK to confirm. Then the curve with new coordinates will display on the screen.

Fig.4-15 Rescale the Coordinate

Switch the Display Status

Click the icon  or  in the Tools Bar to display the spectrum’s upper or latter status. Click  to recover the original status.

Search Peaks

Click the icon  in the Tools Bar to search and label the peaks automatically on the spectrum. The peaks’ height will be listed in the Right Frame. (Fig.4-16)

Click the icon  in the Tools Bar. You should input the peaks’ height that you want to find in the new dialog box and click OK. (Fig.4-17) Then the peaks whose values are smaller than the inputted one will be automatically labeled.

Fig.4-16 Search Peaks

Fig.4-17 Set peak’s height

Math Function

1) Click the icons , ,  or  in the Tools Bar, a new dialog box will display on the screen respectively. You can add, subtract, multiply or divide two spectrums. Let’s take spectrum addition as an example. (Fig.4-18)

Fig.4-18 Spectrum addition source select

Example 4-1

Source Spectrum1 is the spectrum that displayed in the screen, and you can choose Source Spectrum2 in the saved files by clicking the Down arrow. Press OK to confirm, and the result spectrum will be displayed on the screen. (Fig.4-19)

Fig.4-19 Spectrum after addition

2) Click icon  in the Tools Bar, a new dialog box appears on the screen. (Fig.4-20)

Fig.4-20 Spectrum derivative class select

Choose the class (1-10) of derivative, then click OK to confirm. The result spectrum will be display overlaid with the original one. (Fig.4-21)

Fig.4-21 First derivative spectrum

Spectrum Smooth

We use this function to wipe off the interferer of noise during the course of scan, so as to make the spectrum look much more smooth.

Click Spectrum→ Smooth in the main menu.

Multi-wavelength Measurement

We’ll introduce you how to measure a sample in different wavelengths. It always be used in the analysis of unknown compounds.

Measure

- Click the icon  in the Tools Bar, a new form appears on the screen. (Fig.4-22)

Fig.4-22 the main interface of multi wavelength test

- Click the icon  in the Tools Bar to set the parameters.

Fig.4-23 Set parameters

- Choose the quantity of the wavelength you need and input the value of each wavelength.

- Click OK to finish setting.

- Pull the Reference into the light path, click the icon   in the Tools Bar to set zero.

- Pull the sample into the light path, click  to start test. And the test result will be listed in the data sheet. (Fig.4-24)

Fig.4-24 Test Result of multi-wavelength test

Modify a Test Result

1) Click the data frame you want to modify.

2) Pull the sample you want to re-measure in the light path.

3) Click the icon  in the Tools Bar, then the new result will replace the old one.

Delete a Test Result

1) Click the data you want to delete in the data sheet.

2) Click the icon  in the Tools Bar to delete the data.

Name a Sample

1) Select the line you want to rename.

2) Double-click Sample Name, input the sample’s name, press Enter on the keyboard to confirm.

Switch Display Mode

Click the icon  or in the Tools Bar to switch the display mode. (Abs. or Trans.)

DNA/Protein Analysis

We’ll introduce you how to operate a DNA/Protein test.

Measure

- Click the icon  in the Tools Bar to begin a DNA/Protein analysis. (Fig.4-25)

Fig. 4-25 Main interface of DNA/Protein analysis

- Click the icon  to set parameters. (Fig.4-26)

Fig.4-26 Set parameters

3)  Choose measure method first. We provide you 2 methods to measure, and the coefficients will display in the corresponding frames. You can also definite the method by yourself, but you have to input all the coefficients first, then click OK to confirm.

Pls. Ref. Appendix 2

4)  Pull the Reference into the light path, click the icon  in the Tools Bar to set zero.

5)  Pull the sample in the light path, click the icon  to measure. The result will display in

the data sheet. (Fig.4-27)

Fig.4-27 DNA/Protein analysis

Modify a Test Result

1)  Click the data you want to modify in the data sheet.

2)  Pull the sample you want to re-measure in the light path.

3)  Click the icon  to re-measure the sample, and the new value will replace the old one.

Delete a Result

1)  Click the data you want to delete in the data sheet.

2)  Click the icon  in the Tools Bar to delete.

Name a Sample

1) Select the line you want to rename.

2) Double-click Sample Name, input the sample’s name, press Enter on the keyboard to confirm.

#### Other Function

We will introduce you the attached function of M.Wave Professional in this chapter.

Set Wavelength

Click the icon  in the Tools Bar to set wavelength (Fig. 4-28）. Input the value of wavelength in the frame of Set Wavelength, click Goto λ to confirm. Then the instrument will move the wavelength to the point you want, then it blanks automatically.

Fig. 4-28 Set Wavelength

Setup Switch Point of Light Source

Click UV-Spectrophotometer→ Setup Switch Point of Light Source (Fig.4-29), input the value of wavelength, then click OK to confirm.

Fig. 4-28 Set switch point of light source

Turn On/Off W Lamp

Click the button   in the Tools Bar to turn on or turn off the W Lamp.

Turn On/Off D2 Lamp

Click the button  in the Tools Bar to turn on or turn off the Deuterium Lamp.

Set Light Source Switch Mode

Click UV-Spectrophotometer→ Light Source Switch Mode, 3 modes are under your choice.

- Auto Switch Mode --- The instruments will switch automatically at the wavelength point that you set.

- D2 Lamp Mode --- Only the D2 lamp works in the total range of the wavelength.

- W2 Lamp Mode --- Only the W lamp works in the total range of the wavelength.

Set Response Mode

Click UV-Spectrophotometer → Response Mode to choose the corresponding item to set the response speed (Only available in WL. Scan and Kinetics Test).

- Standard Mode—With low speed, but the curve is much more smooth

- Fast Mode--- With fast speed, but the curve is not too smooth.

Get Dark Current

Click UV-Spectrophotometer→ Get Dark Current, the system will re-get the dark current, and the new data will replace the old one.

File Process

We’ll introduce you how to process the test files in this chapter.

List is the file’s styles

Quantity Analysis File        	*.qua

Kinetics Analysis File        	*.kin

Wavelength Scan File       	*.wls

Multi-wavelength File       	*.mti

DNA/Protein Analysis File   	*.dna

Standard Curve            	*.std

Base Line                 	*.sbl

Save Test Result

Click the button   on the Tools Bar, then input the file name and click OK to confirm in the dialog box.

Open a File

Click the button   on the Tools Bar, choose the file name you need in the dialog box, then click Open to confirm.

Print Test Report

Click the button  on the Tools Bar, then set print parameters in the dialog box, press Print to confirm.

#### Appendix 1

Method of Quantitative Analysis

Single Wavelength Method:    Abs. = A1

Double Wavelengths Method:  Abs. = A1-A2

Three Wavelengths Method:   Abs. = A1-(W1-W2)*(A2-A3)/(W2-W3)-A3

#### Appendix 2

DNA/Protein Analysis Method

Method 1

CDNA=(A1-Aref)*f1-(A2-Aref)*f2

CProtein= (A2-Aref)*f3-(A1-Aref)*f4

Ratio=(A1-Aref)/ (A2-Aref)

A1=A260nm，A2=A280nm，Aref=A320nm（Optional）

f1=62.9，f2=36.0，f3=1552，f4=757.3

Method 2

CDNA=(A1-Aref)*f1-(A2-Aref)*f2

CProtein= (A2-Aref)*f3-(A1-Aref)*f4

Ratio=(A1-Aref)/ (A2-Aref)

A1=A260nm，A2=A230nm，Aref=A320nm（Optional）

f1=49.1，f2=3.48，f3=183，f4=75.8